Appendix C

Technical Specifications Schedule

- PRELIMINARY WORKS

  - Architectural plans, structural and mechanical and electrical design.

  - Liability for all statutory permissions and Building Licenses

  - Execution of the construction phase.

  - Supervision of works from qualified Architect and consultant team.

  - Preparation of all legal contracts

  - Houses to be delivered in a clean and completed state.

- EARTHWORKS

  - All excavations and earthworks to be completed in accordance with the approved drawings.

- STRUCTURE

  - Reinforced concrete frame with reinforced concrete raft foundation and external retaining walls, including pool structure, in compliance with current anti-seismic Eurocodes.

- WALLING

  - External walls constructed with 250mm thick blockwork

  - Internal walls constructed with 100mm thick blockwork or metal stud plasterboard partitions.

- WATERPROOFING

  - Mapelastic (Mapei) warrantied waterproofing system, including waterproofing additives (i.e. penetron), water stops and Voltex membrane to be installed to all Basements and Substructures

  - Verandas and wet areas within bathrooms: water proofing membrane (Mapei)

  - Flat Roofs: High performance Felt with pebbles for UV protection

- INTERNAL WALL FINISHES

  - Internal walls to be rendered with three coats of cement mortar and decorated with three coats of spatula and emulsion paint (as per design-white – we plan to go with white - beige color in interiors and change the ultra-white exterior to a soft white-beige color)

- EXTERNAL FINISHES

  - External blockwork and concrete frame to be insulated with self-colored external wall insulation system/polystyrene in accordance with the approved Energy Performance

Certificates [EPC]. Color according to the architect’s design

- FLOORING. Refer to Product Selection Sheet from supplier P David.

a. 60cm x 120cm.Ceramic Floor Tiles to the following areas [Larger dimensions will be charged extra]: Price €30.00 per sq m.

- Entrance Halls and hallways to all levels.

- Kitchen

- Internal Staircases

- W.C’s and Bathrooms

- Balconies and Roof Terraces

- Covered verandas

- Verandas

- Pool tiles, surrounds and balancing tank

- Lift Floor

- 60cm x 60cm.Ceramic Floor Tiles to the following areas:

- Plant Rooms and ancillary areas

Approx.Price €25.00 per sq m.

- SPC composite core flooring suitable for use with underfloor heating, to the following areas:

- Living Rooms

- Dining Rooms

- Bedrooms

Approx.Price €55 per sq m.

- WALL TILES

  - Bathrooms and W.C’s: Ceramic tiles: approx. €30 per sq m all around up to false ceilings.

All dimensions of the tiles should be 60cm x 120cm. Larger dimensions or specialist finishes, will be charged extra.

- EXTERNAL DOORS AND WINDOWS

  - Powder coated double glazed aluminum thermal series sections, with thermal insulating glass in accordance with the requirement set out in the EPC. Provision only for mosquito nets to all habitable rooms.

  - Entrance doors to all Houses will have certification of fire resistant of half hour. Wood finish internally and externally

- GLASS BALUSTRADES at approx. €150 per running meter.

- WARDROBES, KITCHENS CABINETS AND INTERNAL DOORS.

Choice from the showrooms of the preferred suppliers of the vendor. Purchaser may choose from any of the selections of the supplier within the budgeted allowance of €25,000 for 4 bedroom apartment and €20,000 for 3 bedroom apartment. The Purchaser may select materials over the budgeted allowances stated below and bear the cost of the price difference:

        - KITCHENS

- High Quality Design Kitchen Units: Italian design (Lube or similar)

- Worktops and Backsplash. 2cm thick Techno granite or ceramic slab 12mm thickness

- Appliances. Not Included

- Stainless Steel Kitchen Sink and Mixer, provided

        - INTERNAL DOORS

  - 2.4m high, hollow core melamine doors with concealed hinges, magnetic locks, rubber seals, door stops and door handles.

c. WARDROBES

- Wardrobes with doors in designed areas

- SANITARY WARE FITTINGS. Choice from the showrooms of the preferred suppliers of the vendor. Purchaser may choose from any of the selections of the supplier within the budgeted allowance of €4,000 for penthouse and €3,000 for 3 and 4 bedroom apartment, for all the items listed below. The Purchaser may select materials over the budgeted allowances stated below and bear the cost of the price difference):

  - Acrylic Bathtub, with bath screen

  - Wall hung W/C’s with concealed cisterns and integral chrome push buttons.

  - Washbasins with integrated drawer unit

  - Tiled Showers with linear drainage floor gulley and shower screen.

  - Chrome Italian Taps and Mixers.

  - Supply of Bathroom Mirrors and accessories are not included. Power provisions for illuminated mirrors is to be provided.

  - Rail heat towel provisions

- PLUMBING-DRAINAGE

  - Hot & Cold-water installation includes plastic storage tanks high-pressure system (pipe in pipe) and manifolds on each floor.

  - Hot water circulation pump

  - ELCORA Hot water solar collectors to roofs.

  - Provision only for Water Softener.

  - Underfloor heating with associated electric Hisense (or similar brand) outdoor heat pump, located on roofs.

  - Mechanical ventilation, provided to kitchen extractors and all bathrooms without an openable window.

  - Below Ground Drainage and sewage system connected to drainage pits and to the central sewage system of Limassol respectively.

- ELECTRICAL

  - Concealed circuits in accordance with current IEE Electrical Installations and regulations as required by the Cyprus Electrical Authority. To include an adequate number of power supply points and switches as per electrical drawings, comprising of circuits for lighting, power points, data sockets, telephone points and waterproof sockers where applicable. (extra electrical points may be subject to extra charge accordingly)

  - To provide basic light fittings throughout. Decorative pendants and light fittings not included.

  - Photovoltaics are for the common spaces.

  - Provision only for one electric car charging point (one at whole parking area)

  - Provide Basic KNX Smart Home Automation System for lighting and underfloor heating. System functions can be expanded, at additional cost.

  - Security Alarm provisions only.

- AIR CONDITIONING

  - Installation of Hisense (or similar) a/c split units to all habitable rooms.

- FALSE CEILINGS. Gypsum suspended ceiling to be provided on all bathrooms and corridors to hide ducted AC units. Suspended ceiling can be provided in the whole apartment upon request and additional cost

- THERMAL INSULATIONS

  - According to EPC study

- LIFT FOR EACH APARTMENT BUILDING. To access four levels (-1, 0, 1, 2)

- COMMON EXTERNAL AREAS & PARKING AREA

  - Cobblestone with appropriate painted road marking to designate the parking space of each house.

  - Low Maintenance plants and shrubbery with provisions for irrigation system.

- ENTRANCE

  - Provide access via the two existing complex security gates.

  - CCTV camera door phone.

- SWIMMING POOL with waterfall and balancing tank, as per architect’s design. Following items will not be included (jacuzzi jets, heat pump and pool cover). Private jacuzzi to be placed on the penthouses (at an additional cost)

- LANDSCAPING

  - Common Parts to be landscaped.

  - Private Gardens to be handed over to client with planters filled with topsoil and provided with adequate watering points. Irrigation system and planting is not included.

  - Tree planting is not included

`